# Calibration Program Method Guideline

# Prerequisites

Before you begin, ensure the following are prepared and ready:

- A circle grid calibration board (BDB-6) securely mounted on the cobot TCP.

- A working camera connected to the system and positioned to view the TCP area.

- An ARC Cobot connected and controllable via network.

- The Python environment and all required dependencies installed.

# Step-by-Step Guide:

This section explains how to configure and run the collect_calibration_data.py script to capture images for calibration purposes.

## Collecting Calibration Data

This guide explains how to configure and run the collect_calibration_data.py script to capture images for camera calibration.

### Step 1: Open the Script

Locate and open the file:	collect_calibration_data.py

### Step 2: Update Configuration Settings

Inside the script, modify the following parameters:

- File Name for Saved Images

Set the filename or folder where captured images will be stored.

- Camera IP Address

Update the IP to match your connected camera device.

- Robot and PC IP Address

Set the pc and robot’s IP address for motion control.

Examples:

- File name 		  : 	mechmind_logMleft_25jun

- Camera IP address  : 	192.168.100.205

- Robot IP address     : 	192.168.100.210

- PC IP address 	  : 	192.168.100.200

### Step 3: Define Starting Pose

Physically move the robot so that:

- The calibration board is centered in the camera’s field of view.

- The Z-axis is perpendicular (normal) to the board.

Update the starting pose (XYZ and orientation) in the script:

This point will be used as the initial pose from which automated path movement begins.

Example :

Robot starting position : 	100, -600, -100

### Step 4: Run the Script

Once all settings are configured, run the script

### Step 5: Verify Captured Images

The script will automatically capture and save images as the robot moves.

Check the saved files in the folder defined earlier (e.g., mechmind_logMleft_25jun) to ensure:

- All circles on the calibration target are clearly visible.

- The depth map of the circle grid is present and correctly rendered.

### Step 6: Adjust as Needed

The image capture path is generated automatically.

If the board moves out of view or circles are not properly visible:

- Press the E-stop to halt the robot immediately.

- Adjust the robot’s starting position or orientation.

- Update the corresponding starting pose in the script.

## Calibration grid selecting

This guide explains how to use calibration_grid_select_blob.py to label the calibration circles in the captured images for camera calibration.

### Step 1: Open the Script

Locate and open the file: 	calibration_grid_select_blob.py

### Step 2: Update the Image Folder Path

In the script, modify the path to point to the folder containing the saved calibration images.

Example :

Image folder 	:	mechmind_logMright_25jun

### Step 3: Run the Script

Once all settings are configured, run the script

### Step 4: Select Calibration Circles

For each image that appears:

- The script will automatically detect and highlight all detected circles.

- To select the calibration grid:

- Click and drag with the left mouse button to draw a rectangle around the grid area.

- Ensure that only the calibration circles are included in the region.

- check the sequence numbers — they should start from the top-left circle and follow a row-wise order (left to right, then top to bottom).

⚠️ Important:

If the region contains too few or extra circles, the script will prevent you from proceeding to the next step. In that case, reselect the region correctly.

### Step 5: Select Top-Left Corner Circle

After selecting a valid region:

- Click on the top-left circle of the calibration board as shown in the previous image

- The top-left circle refers to the one located on the side with three circles and is the farthest from the TCP.

- This defines the orientation of the grid.

If you selected the wrong point:

- Right-click to restart the labeling for that image.

### Step 6: Proceed or Skip

Once the region and corner are correctly selected:

- Press the spacebar to proceed to the next image.

If an image is incomplete (e.g., not all circles are visible):

- You may skip the image by pressing the spacebar without labeling.

## Calibration grid selecting

### Step 1: Open the Script

Locate and open the file: 	run_computation_scripts

### Step 2: Update the Image Folder Path

In the script, modify the path to point to the folder containing the saved calibration images.

Example :

Image folder 	:	mechmind_logMright_25jun

Copy and paste a camera.npz file based on the type of camera used from manufacturer_camera_parameters into the data file and rename it as camera_parameters.npz

### Step 3: Run the Script

Once all settings are configured, run the script

After execution, three files will be generated in the output folder:

- optimized.npz

- board_alignment.npz

- extrinsic.npz

Take the optimized.npz file, rename it appropriately, and save it into the camera_parameters folder. This completes the calibration process.